UIOFRST   UIOF Format Reset Utility

Copyright Summagraphics Corporation 1991.  All Rights Reserved.

UIOFRST transmits a reset command (<ESC> Z) to the tablet out the

selected serial port.

Usage:  UIOFRST [options]

/1	  selects serial port 1 (PC, XT, AT, PS/2)

/2	  selects serial port 2 (PC, XT, AT, PS/2)

/3	  selects serial port 3 (PS/2 only)

/4	  selects serial port 4 (PS/2 only)

/5	  selects serial port 5 (PS/2 only)

/6	  selects serial port 6 (PS/2 only)

/7	  selects serial port 7 (PS/2 only)

/8	  selects serial port 8 (PS/2 only)

/BAxxxx	  selects serial port with base address of xxxx (in HEX)

/N	  selects no parity configuration

/? or /H  displays help screen

NOTES:  - When using the BA option (Base Address), all four digits must

be specified.

- This program transmits in the UIOF/Microgrid default serial

communication configuration:  9600 baud, 7 data bits, even parity,

and two stop bits.

- Use the /BA switch only when the serial port is located at a

non-standard address within the computer.

- A summary of command line switches is always available by starting

the program with switch /? or /H.

Serial Port Address Table

Command Line	Serial	   Serial Port	 System

Switch		Port	   Address

------------------------------------------------------------------------------

/1		1	   03F8 Hex	 All (PC, XT, AT, 386, and any PS/2)

/2		2	   02F8 Hex	 All (PC, XT, AT, 386, and any PS/2)

/3		3	   3220 Hex	 Any PS/2

/4		4	   3228 Hex	 Any PS/2

/5		5	   4220 Hex	 Any PS/2

/6		6	   4228 Hex	 Any PS/2

/7		7	   5220 Hex	 Any PS/2

/8		8	   5228 Hex	 Any PS/2

------------------------------------------------------------------------------

*********************** 87-4007-001 ***************************