TABLET.COM(TM) is a tablet driver which can emulate Microsoft(R)'s

MOUSE.COM driver. TABLET.SYS is a system file that can be used in your

CONFIG.SYS file. TABLET.COM is a TSR (Terminate and Stay Resident) program

that can be run from the DOS command prompt or from a batch file, including

autoexec.bat.

TABLET.SYS and TABLET.COM should  NOT  be loaded at the same time.

Usage: DEVICE=TABLET.SYS [Com Port] [Pointing Device] [Options]

or

TABLET.COM [Com Port] [Pointing Device] [Options]

Where: Com Port = /1       for Com Port 1   <DEFAULT>

/2       for Com Port 2

/3       for Com Port 3, PS/2 models only

/4       for Com Port 4, PS/2 models only

/BAxxxx  selects serial port with base address xxxx (in HEX)

/Ix      selects serial port with IRQ line number x

(allowable IRQ 2 through 7)

Pointing

Device =   /C4      for four-button cursor

/C3	   for stylus with two barrel buttons

/CP      for stylus with one barrel button  <DEFAULT>

OPTIONS:          /OFF  to disable the driver

/A    Absolute Tracking

/R    Relative Tracking  <DEFAULT>

(Following four options work in relative mode only:)

/S##  Sets both horizontal and vertical sensitivity -

Range = 0 - 99, Default = 50

/H##  Sets only the horizontal sensitivity -

Range = 0 - 99, Default = 50

/V##  Sets only the vertical sensitivity -

Range = 0 - 99, Default = 50

/M#   Ballistic Gain Profile.  Controls acceleration

of the screen cursor.  The driver comes with

4 built-in ballistic gain profiles: Slow,

Moderate, Fast, and Unaccelerated. Specify 1,

2, 3, or 4, respectively.

/CM####  allows a user-defined button mapping

The /CM argument is followed by 1-4 digits that

specify which tablet switches are assigned to

standard mouse buttons.  Each digit represents

the value of a tablet switch, and its position

represents the mouse button that it is assigned

to: /CMlrmb where "l" is replaced by the number

of the switch that should be translated to the

left mouse button; "r" is the right mouse button

switch, "m" is the middle button switch, and "b"

is a switch that will emulate both left and right

mouse buttons being pressed simultaneously.

An example:

TABLET /CM2134

would map tablet switch #2 to the left mouse button,

switch #1 to the right mouse button, switch #3 to

the middle button and switch #4 to both left and

right mouse buttons.

/O#   The /O option overrides an application that changes

the mouse cursor for text modes by "locking" the

mouse cursor to the default text cursor.  Use

/O1 to enable locked cursor;

/O0 to disable locked cursor <DEFAULT>

/N### The /N option specifies how many screen updates to

skip before actually plotting the cursor.  This

feature is useful for Laptops and other systems

with slower LCD displays.  A small value is recom-

mended because driver performance can be affected.

### is a number from 0-255.

/N0	<DEFAULT>

NOTES:		    - When selecting a serial port other than 1,2,3 or 4, the

/BA and /I options must be used together.

- When using the /BA option, all four digits must be

specified (Example:  for Com 1 - /BA03F8)

- Use the /BA and /I options only when a non-standard

serial address is used in the computer.

Serial Port Address Table

Command Line	Serial	   Serial Port	 System

Switch		Port	   Address

------------------------------------------------------------------------------

/1		1	   03F8 Hex	 All (PC, XT, AT, 386, and any PS/2)

/2		2	   02F8 Hex	 All (PC, XT, AT, 386, and any PS/2)

/3		3	   3220 Hex	 Any PS/2

/4		4	   3228 Hex	 Any PS/2

------------------------------------------------------------------------------

CHANGING AN INSTALLED TABLET DRIVER'S PARAMETERS

------------------------------------------------

Command-line options can also be issued after the driver is initially

installed by running TABLET.COM again.  For example, you can go back and

change the sensitivity of the cursor tracking with the /S command option

at any time.  It is possible to change the parameters of an installed

TABLET.SYS driver using TABLET.COM.  The TABLET drivers will not install

twice, as long as they are the same version.

************************* 87-1005-005 *************************************