ECP PRODUCTION TEST

VERSION 3.00

USER MANUAL

June 9, 1998

Copyright ã 1995-1998 Standard Microsystems Corporation

Table of Contents

INTRODUCTION

PROGRAM DESCRIPTION

SMC Device Identification

ECP Port Recognition

Bus Interface Tests

Parallel Port Transaction Tests

Cable Connection Test

Bi-Directional Test

EPP Test

ECP Test

RUNNING THE ECP PRODUCTION TEST SOFTWARE

THE ECP300.INI FILE

Description

Commands

DEVICE

CABLE_TESTS

TEST_COUNT

DMA_CHANNEL

IRQ_CHANNEL

EPP_TEST

PAUSE_ON_PASS

PAUSE_ON_FAIL

ECP300.EXE ERROR MESSAGES

# INTRODUCTION

·	The SMC ECP Production Test software, ECP300.EXE, along with an EVB34C60 or equivalent SMC34C60-based parallel port peripheral can test the functionality of the Extended Capabilites Port in SMC's Super I/O and Ultra I/O controllers.

·	ECP300.EXE, which runs under MS-DOS and Microsoft Windows 3.1, tests the ECP FIFO, controller pins A10, PDRQ, PDACK, and PINTR, and data transfers in Bi-Directional, EPP (1.9), and ECP Mode.

·	If all of the tests are completed successfully, ECP300.EXE terminates with a large PASS banner and beeps once.  If one or more tests fail, the program displays a large FAIL banner and beeps four times.  Failures also generate a test summary that is printed following the program banner.

·	Operational parameters for this software can only be controlled through the commands in the ECP300.INI file.  The one allowable command line argument, -?, prints the ECP300.EXE version number followed by a short program description and SMC contact information.

# PROGRAM DESCRIPTION

The ECP Production Test consists of two basic test functions that are performed in four distinct phases.  The device phase and port phase of the identification function confirm that an appropriate operating environment has been established.  The internal phase and external phase of the ECP testing function verifies the port integrity.

## SMC Device Identification

The device identification phase verifies that an SMC Super or Ultra I/O device is present.  The ECP Production Test will only run on platforms that incorporate one of these SMC devices.  See the ECP300.INI file DEVICE command on page 10.

## ECP Port Recognition

If a valid SMC ECP controller device is detected, the ECP Production Test checks for active ECP ports at addresses 278H, 378H, or 3BCH.  The port address must be either 278H or 378H if EPP testing is enabled (see the ECP300.INI file EPP_TEST switch on page 10).  Note: ECP300.EXE tests only the last ECP port found in the system.

## Bus Interface Tests

The DMA and FIFO Bus Interface Tests are the first phase of ECP testing.  These tests read and write the ECP FIFO to exercise the internal Host data paths, DMA, Programmed I/O, and Interrupt functions.  The Bus Interface Tests are always performed, regardless of the state of the ECP300.INI file CABLE_TESTS switch (see page 10).

## Parallel Port Transaction Tests

The final phase of ECP testing performs four transaction tests to exercise the external parallel port interface.  These tests are listed below.  The Parallel Port Transaction Tests may or may not be performed depending on the state of the ECP300.INI file CABLE_TESTS switch (see page 10).

### Cable Connection Test

The Cable Connection Test verifies the parallel port cable connection to the EVB34C60 or SMC34C60-based parallel port peripheral using the MAP Protocol daisy chain port functions.

### Bi-Directional Test

The Bi-Directional Test transmits and receives 256 bytes of data between the PC host and the parallel port peripheral in Bi-Directional mode using the SMC34C60 Port Test Read and Port Test Write registers.

### EPP Test

If enabled, the EPP Test transmits and receives 256 bytes of data between the PC host and the parallel port peripheral in EPP mode using the SMC34C60 Port Test Read and Port Test Write registers.  The EPP Test can be disabled using the ECP300.INI file EPP_TEST switch (see page 10).

### ECP Test

The ECP Test transmits and receives 256 bytes of data between the PC host and the parallel port peripheral in ECP mode using the SMC34C60 Port Test Read and Port Test Write registers.

# RUNNING THE ECP PRODUCTION TEST SOFTWARE

·	ECP300.EXE does not configure the SMC controller for ECP Mode. Before running this program make sure that the ECP parallel port is properly initialized.  You can do this using programs like CRE52.EXE or 665.EXE that configure the SMC Super I/O devices.  These programs are available on the SMC Web site WWW.SMC.COM.

·	Edit the ECP300.INI file for the required ECP Production Test configuration (see THE ECP300.INI FILE section on page 8) and connect an EVB34C60 or equivalent SMC34C60-based parallel port peripheral to the test host.

·	Run ECP300.EXE.  If the number of test iterations is greater than one (see TEST_COUNT on page 10), type Esc at any time to abort testing.

# THE ECP300.INI FILE

## Description

·	The commands in the ECP300.INI file (Figure 1) establish the various operating parameters for the ECP Production Test software.

·	The ECP300.INI file commands are case sensitive and must be followed by a numeric argument.  There can be only one ECP300.INI file command per line.

·	Comments in the ECP300.INI file follow semicolons.  Comments may not preceed commands on the same line, nor may they separate a command from its argument.

·	The ECP300.INI must be located in the same directory as the ECP300.EXE file.  If the ECP Production Test cannot find the ECP300.INI file, a new file is created, automatically.

;

; ECP300.INI file

; 8/30/95

;

; COMMAND        VALUE  DESCRIPTION

;-------------------------------------------------------------------

DEVICE           0      ;Device To Identify (665, 666, 669=669&669FR

;931, 932 = 93x & 93xFR, 951 = 95x & 95xFR

;602=60x, 612=61x, 769, 702=70x,772=77x,782=78x,

;672=67x, 682=68x,0 = Look For Any Supported ;Device)

CABLE_TESTS      1      ;34C60 Tests Switch (0-1, default 1)

;0 = Only ECP DMA & FIFO Tests

TEST_COUNT       1      ;Test Iterations (1-32000, default 1)

DMA_CHANNEL      3      ;DMA Channel Number (0-3, default 3)

IRQ_CHANNEL      7      ;IRQ Channel Number (1-7, default 7)

EPP_TEST         1      ;EPP Test Enable Switch (0-1, default 1)

PAUSE_ON_PASS    0      ;Pause on Pass Switch (0-1, default 0)

PAUSE_ON_FAIL    0      ;Pause on Fail Switch (0-1, default 0)

Figure 1 - ECP300.INI

## Commands

### DEVICE

DEVICE provides support for multiple SMC FDC37CXXX devices.  The DEVICE argument represents the specific device ECP300.EXE will try to identify.  Valid DEVICE argument values include 602, 612,665, 666, 669, 702, 772, 769, 782, 931, 932 and 951.  If the DEVICE argument is 0, ECP300.EXE will try to identify any of the currently supported devices.  The default is 0; search for all supported.

### CABLE_TESTS

CABLE_TESTS identifies the type of tests ECP300.EXE will execute.  If the CABLE_TESTS argument is 0, only the Bus Interface Tests are performed.  If the CABLE_TESTS argument is 1, all of the ECP Production Tests are performed; including, the Bus Interface Tests and the Parallel Port Transaction Tests.  The default is 1.

### TEST_COUNT

TEST_COUNT specifies the number of test iterations performed before ECP300.EXE terminates.  The TEST_COUNT argument can be from 1 to 32,000, the default is 1.  Test iterations can be aborted at any time by typing Esc.

### DMA_CHANNEL

DMA_CHANNEL specifies the DMA channel number for the DMA Bus Interface Test.  The DMA_CHANNEL argument can be from 0 to 3, the default is channel 3.

### IRQ_CHANNEL

IRQ_CHANNEL specifies the hardware interrupt level for the ECP FIFO and DMA Bus Interface Tests.  The IRQ_CHANNEL argument can be from 1 to 7, i.e. IRQ1 to IRQ7.  The default is 7.

### EPP_TEST

EPP_TEST enables the EPP Parallel Port Transaction Test if the argument is 1.  The EPP_TEST argument can be 0 or 1, the default is 1.  An error will occur if the ECP Port Address is 3BCH when this test is enabled.

### PAUSE_ON_PASS

PAUSE_ON_PASS halts ECP300.EXE program iterations, i.e. test counts greater than one, when the PAUSE_ON_PASS argument is 1 and all Bus Interface and Parallel Port Transaction Tests for the current test iteration have passed.  The PAUSE_ON_PASS argument values can be 0 or 1, the default 0.

### PAUSE_ON_FAIL

PAUSE_ON_FAIL halts ECP300.EXE program iterations, i.e. test counts greater than one, when the PAUSE_ON_FAIL argument is 1 and one or more Bus Interface and Parallel Port Transaction Tests for the current test iteration have failed.  The PAUSE_ON_FAIL argument values can be 0 or 1, the default 0.

# ECP300.EXE ERROR MESSAGES

·	"No interrupt after DMA"

·	"DMA test compare error"

·	"No FIFO test interrupt"

·	"FIFO test compare error"

·	"Invalid EPP base address (3BCH)"

·	"External parallel port device not connected"

·	"Parallel port SPP READ error"

·	"Parallel port EPP READ error"

·	"Parallel port ECP READ error"

·	"Parallel port SPP WRITE error"

·	"Parallel port EPP WRITE error"

·	"Parallel port ECP WRITE error"