DGMG.COM is an ADI driver that allows you to use your 16-button SummaSketch

tablet with AutoCAD(R),Autoshade(R), and Autosketch(R) by AutoDesk, Inc.

DGMG.COM provides full ADI v4.1 compatibility.

Within AutoCAD select the ADI driver option for the digitizer driver.

The Summagraphics ADI driver is always installed on Interrupt 79

(hexadecimal), which is the AutoCAD default.

Loading the ADI Driver:

DGMG has several options:

DGMG [/#] [/C#] [/A#]

The first argument above selects the serial port that the tablet

is connected to.  If this argument is omitted the driver will be

installed on the first serial port it finds.

/1 selects com1

/2 selects com2

/3 and /4 selects com3 and com4 on PS/2 systems.

The second argument selects the cursor (or stylus) type that

you will be using with your UIOF-format tablet.  There are two

possibilities:

/CP selects a stylus tranducer

/C16 selects a 16-button cursor

The default transducer type is a stylus.  If you have more than one

type of transducer attached to your tablet, the /C# command line

argument will select which transducer the driver will use.

The third argument allows digitizing paper drawings in AutoSketch.

/AC for resetting to the ADI specification

/AS selects AutoSketch digitizing mode

The default mode is the standard AutoCAD (/AC) mode which uses

the full active area of the tablet.  The AutoSketch mode was

designed for AutoDesk's AutoSketch drawing program.  It scales

the digitized coordinates from the tablet so that one inch measured

on the tablet surface corresponds to one inch measured or plotted

by AutoSketch.

********************** 87-4009-003 **************************